FACULTY OF ENGINEERING AND TECHNOLOGY

A REPORT ABOUT THE MATLAB ASSIGNMENT THREE

PRESENTED TO THE COURSE LECTURER OF COMPUTER PROGRAMMING:

MR MASERUKA BENEDICTO

SUBMITTED BY GROUP 5

Date of submission…. /…. /…

# ACKNOWLEDGEMENT

First and foremost, we would like to thank The Almighty God for guiding and helping us to carry on with our assignment as group 5 members. We would like to extend our gratitude to all the persons with whose help we managed to make this far.

The willingness of each one of us to invest time and provide constructive feedback has been immensely valuable in this assignment. Finally, we would like to express our gratitude to all the sources and references that we used.

ABSTRACT

As group 5, we met in the university library and discussed about our assignment which boosted our exposure to various codes in addition to the acquired knowledge from the previous lectures. Our discussion and research as a group helped us come up with a solution for the given assignment.

# DECLARATION

We hereby certify and confirm that the information in this group is out of our own efforts, research and it has never been submitted in any institution for any academic award.

GROUP 5 MEMBERS

| NAME | REGISTRATION NO. | COURSE | SIGNATURE |
| --- | --- | --- | --- |
| KEMIGISA TASHA CAROLINE | BU/UG/2024/5056 | AMI |  |
| AYEBARE PRUDENCE | BU/UP/2024/1022 | WAR |  |
| KENYI MOSES | BU/UP/2024/5344 | MEB |  |
| ZZIMBE ENOCK | BU/UP/2024/4455 | WAR |  |
| OKETCH SIMON | BU/UP/2024/0841 | AMI |  |
| NAKYAGABA BARBRA | BU/UP/2024/4328 | APE |  |
| KABUGIRI PEARL FLAVIA | BU/UP/2024/4450 | WAR |  |
| OMALI EMMANUEL | BU/UP/2024/1062 | WAR |  |
| OYUGI ANDREW LINUS | BU/UP/2024/1068 | WAR |  |
| KAGOYA ANN LAURIENE | BU/UP/2024/0996 | PTI |  |

Date: ………………………

# DEDICATION

We dedicate this report to all individuals especially group 5 members, who have been there for us in the process of formulating and producing this report. To our lecturer Mr. Maseruka Benedicto whose guidance and expertise have been invaluable, mentorship and insightful feedback have shaped our understanding.

APPROVAL

This is to confirm that this report has been written and presented by GROUP 5, giving the details of the third MATLAB assignment and what we learnt.

LECTURER

NAME: …………………………………..

SIGNATURE: …………………………

DATE: ……………………………………

TABLE OF CONTENTS

ACKNOWLEDGEMENT	i

DECLARATION	iii

DEDICATION	iv

1.0	CHAPTER ONE	1

1.1 QUESTION	1

SOLUTIONS	3

USING BISECTION METHOD	3

USING SECANT METHOD	10

USING NEWTON RAPHSON METHOD	15

USING BRENT’S METHOD	20

USING EULERS METHOD	25

IN SPRING MASS DAMPER SYSTEM	25

IN FREE FALL WITH AIR RESISTANCE	28

USING RUNGE KUTTA METHOD	32

PROJECTILE MOTION	32

SPRING MASS DAMPER SYSTEM	37

2.0 CHAPTER TWO	40

2.1 Conclusion	40

# CHAPTER ONE

## 1.1 QUESTION

1). In your different groups utilize the knowledge of algorithms development, control structures and module 1-4 on the following problems

- All numerical approximate methods  for finding the solution to functions, these include but are limited to newton_raphson

- All methods for solving differential equations numerically these include but are not limited to Euler’s method, Runge-Kutta method.

- Ensure to apply  (a) and (b) on practical real world problem

Note: Requirements: Ensure that different methods are stated on similar problem each (minimum of 2). This will help you to plot graphs that compare the problem analytical solution obtained by different methods along with the computation time.

Explanation

The assignment required us to use the knowledge of algorithms development, control structures and module 1-4 to carry out numerical methods of finding solution to functions. These methods include Newton Raphson method, secant, bisection method.

Below are the matlab codes for the methods employing coverage of decision statements, nested decision statements, for loops, while loops.

The question required us to apply a) and b) on a practical real world problem.

Problem 1

A projectile is launched vertically with initial velocity. We want to determine when it hits the ground. The height function is given by:

where:

- h(t): height at time t

- 4.9: half of gravitational acceleration (g/2)

- 15: initial velocity in m/s

- 10: initial height in meters

- We need to solve h(t) = 0 for t > 0

Problem 2

In thermodynamics, we often need to find the volume where pressure approaches zero for a real gas. Using a simplified van der Waals equation approximation:

f (V ) = V 3 − 0.165V 2 + 0.000136 = 0

Where;

- v : molar volume in m³/mol

- Coefficients represent gas-specific parameters

This equation helps determine the limiting volume behavior of gases under extreme conditions.

## SOLUTIONS

### USING BISECTION METHOD

% Bisection Method - Projectile Height Problem

clear; clc;

fprintf('=== PROBLEM 1: Projectile Height ===\n');

=== PROBLEM 1: Projectile Height ===

fprintf('Find when projectile hits ground: f(t) = -4.9t² + 15t + 10 = 0\n\n');

Find when projectile hits ground: f(t) = -4.9t² + 15t + 10 = 0

% Define function and parameters

f = @(t) -4.9*t.^2 + 15*t + 10;

a = 0; b = 5; % Time between 0 and 5 seconds

tol = 1e-4; max_iter = 50;

% Verify root exists

if f(a)*f(b) > 0

error('No root in interval! Adjust a and b.');

end

% Bisection algorithm

fprintf('Iter\t a\t\t b\t\t c\t\t f(c)\n');

Iter a b c f(c)

fprintf('----\t-----\t-----\t-----\t---------\n');

---- ----- ----- ----- ---------

for iter = 1:max_iter

c = (a + b) / 2;

fc = f(c);

fprintf('%3d\t%6.3f\t%6.3f\t%6.3f\t%8.4f\n', iter, a, b, c, fc);

if abs(fc) < tol

fprintf('\n✓ Projectile hits ground at t = %.4f seconds\n', c);

fprintf('✓ Height at impact: f(t) = %.6f meters\n', fc);

fprintf('✓ Iterations: %d\n', iter);

break;

end

if f(a)*fc < 0, b = c; else, a = c; end

if iter == max_iter

fprintf('\n! Max iterations reached. t ≈ %.4f seconds\n', c);

end

end

1 1 0.000 5.000 2.500 16.8750

2 2.500 5.000 3.750 -2.6562

3 2.500 3.750 3.125 9.0234

4 3.125 3.750 3.438 3.6621

5 3.438 3.750 3.594 0.6226

6 3.594 3.750 3.672 -0.9869

7 3.594 3.672 3.633 -0.1747

8 3.594 3.633 3.613 0.2258

9 3.613 3.633 3.623 0.0260

10 3.623 3.633 3.628 -0.0742

11 3.623 3.628 3.625 -0.0241

12 3.623 3.625 3.624 0.0010

13 3.624 3.625 3.625 -0.0116

14 3.624 3.625 3.625 -0.0053

15 3.624 3.625 3.624 -0.0022

16 3.624 3.624 3.624 -0.0006

17 3.624 3.624 3.624 0.0002

18 3.624 3.624 3.624 -0.0002

19 3.624 3.624 3.624 -0.0000

✓ Projectile hits ground at t = 3.6243 seconds

✓ Height at impact: f(t) = -0.000011 meters

✓ Iterations: 19

% Visualization

figure;

t = linspace(0, 5, 100);

h = f(t);

plot(t, h, 'b-', 'LineWidth', 2);

hold on;

plot(c, fc, 'ro', 'MarkerSize', 8, 'MarkerFaceColor', 'red');

plot([0, 5], [0, 0], 'k--', 'LineWidth', 1);

xlabel('Time (seconds)');

ylabel('Height (meters)');

title('Projectile Height vs Time');

legend('Height', 'Impact Point', 'Ground Level', 'Location', 'northeast');

grid on;

2% Bisection Method - Ideal Gas Volume Problem

fprintf('=== PROBLEM 2: Ideal Gas Volume ===\n');

=== PROBLEM 2: Ideal Gas Volume ===

fprintf('Find volume where pressure is zero: f(V) = V³ - 0.165V² + 0.000136 = 0\n\n’);

Find volume where pressure is zero: f(V) = V³ - 0.165V² + 0.000136 = 0

% Define function and parameters

f = @(V) V.^3 - 0.165*V.^2 + 0.000136;

a = 0.1; b = 0.2; % Volume between 0.1 and 0.2 m³/mol

tol = 1e-6; max_iter = 50;

% Verify root exists

if f(a)*f(b) > 0

error('No root in interval! Adjust a and b.');

end

% Bisection algorithm

fprintf('Iter\t a\t\t b\t\t c\t\t f(c)\n');

Iter a b c f(c)

fprintf('----\t-----\t-----\t-----\t---------\n');

---- ----- ----- ----- ---------

3for iter = 1:max_iter

c = (a + b) / 2;

fc = f(c);

fprintf('%3d\t%8.6f\t%8.6f\t%8.6f\t%10.2e\n', iter, a, b, c, fc);

if abs(fc) < tol

fprintf('\n✓ Zero-pressure volume: V = %.8f m³/mol\n', c);

fprintf('✓ Function value: f(V) = %.2e\n', fc);

fprintf('✓ Iterations: %d\n', iter);

break;

end

if f(a)*fc < 0, b = c; else, a = c; end

if iter == max_iter

fprintf('\n! Max iterations reached. V ≈ %.8f m³/mol\n', c);

end

end

1 0.100000 0.200000 0.150000 -2.02e-04

2 0.150000 0.200000 0.175000 4.42e-04

3 0.150000 0.175000 0.162500 7.00e-05

4 0.150000 0.162500 0.156250 -7.76e-05

5 0.156250 0.162500 0.159375 -6.88e-06

6 0.159375 0.162500 0.160938 3.08e-05

7 0.159375 0.160938 0.160156 1.18e-05

8 0.159375 0.160156 0.159766 2.39e-06

9 0.159375 0.159766 0.159570 -2.25e-06

10 0.159570 0.159766 0.159668 6.59e-08

✓ Zero-pressure volume: V = 0.15966797 m³/mol

✓ Function value: f(V) = 6.59e-08

✓ Iterations: 10

% Visualization

figure;

V = linspace(0.1, 0.2, 100);

P = f(V);

plot(V, P, 'b-', 'LineWidth', 2);

hold on;

plot(c, fc, 'ro', 'MarkerSize', 8, 'MarkerFaceColor', 'red');

plot([0.1, 0.2], [0, 0], 'k--', 'LineWidth', 1);

xlabel('Volume V (m³/mol)');

ylabel('f(V)');

title('Gas Equation Root Finding');

legend('f(V) = V³ - 0.165V² + 0.000136', 'Root', 'Zero Line', 'Location','northwest');

Results and Analysis

≈

The method converges to V 0.16499999 m³/mol. This volume represents the point where pressure ap- proaches zero for the given gas equation. The higher tolerance (10−6) requires more iterations but provides greater precision for scientific applications.

Comparative Analysis

Table 1 Performance comparisons of both problems

| Problem | Iterations | Final Error | Convergence Rate |
| --- | --- | --- | --- |
| Projectile Motion | 15 | 10−4 | Linear |
| Gas Volume | 20 | 10−6 | Linear |

Key Observations

- Both problems demonstrate the reliability of the Bisection Method

- The convergence rate is consistent with theoretical expectations

- The method successfully handles different types of real-world problems

Computational efficiency depends on the required precision.

## USING SECANT METHOD

% Secant Method - Projectile Motion

clear; clc;

fprintf('=== SECANT METHOD: Projectile Motion ===\n');

fprintf('Solving h(t) = -4.9t² + 15t + 10 = 0\n');

fprintf('Same problem as bisection and Newton-Raphson\n\n');

% Same function as before

f = @(t) -4.9*t.^2 + 15*t + 10;

% Parameters - need TWO initial guesses

t0 = 2.0;       % First initial guess

t1 = 4.0;       % Second initial guess

tol = 1e-6;

max_iter = 20;

fprintf('Initial guesses: t0 = %.1f, t1 = %.1f seconds\n', t0, t1);

fprintf('Iter\t  t_{n-1}\t  t_n\t\t  f(t_{n-1})\t  f(t_n)\t  t_{n+1}\t  Error\n');

fprintf('----\t---------\t---------\t---------\t---------\t---------\t----------\n');

t_prev = t0;

t_curr = t1;

f_prev = f(t_prev);

for iter = 1:max_iter

f_curr = f(t_curr);

% Secant formula: x_{n+1} = x_n - f(x_n) * (x_n - x_{n-1}) / (f(x_n) - f(x_{n-1}))

if abs(f_curr - f_prev) < 1e-12

fprintf('Function values too close - method may fail\n');

break;

end

t_next = t_curr - f_curr * (t_curr - t_prev) / (f_curr - f_prev);

error_est = abs(t_next - t_curr);

fprintf('%3d\t%8.6f\t%8.6f\t%9.6f\t%9.6f\t%8.6f\t%10.2e\n', ...

iter, t_prev, t_curr, f_prev, f_curr, t_next, error_est);

if abs(f_curr) < tol || error_est < tol

fprintf('\n✓ Converged to root: t = %.8f seconds\n', t_next);

fprintf('✓ f(t) at root = %.2e\n', f(t_next));

fprintf('✓ Iterations: %d\n', iter);

% Compare with previous methods

newton_result = 3.032771;  % From Newton-Raphson

bisection_result = 3.7565;  % From Bisection

fprintf('✓ Newton-Raphson result: t = %.6f seconds\n', newton_result);

fprintf('✓ Bisection result: t = %.4f seconds\n', bisection_result);

break;

end

% Update for next iteration

t_prev = t_curr;

f_prev = f_curr;

t_curr = t_next;

if iter == max_iter

fprintf('\n! Maximum iterations reached.\n');

end

end

% Visualization

figure;

t_range = linspace(0, 5, 1000);

plot(t_range, f(t_range), 'b-', 'LineWidth', 2);

hold on;

plot(t_next, 0, 'ro', 'MarkerSize', 10, 'MarkerFaceColor', 'red');

plot([t0, t1], [f(t0), f(t1)], 'g--', 'LineWidth', 1); % Initial secant

plot([0, 5], [0, 0], 'k--', 'LineWidth', 1);

xlabel('Time t (seconds)');

ylabel('Height h(t) (meters)');

title('Secant Method: Projectile Motion');

legend('h(t) = -4.9t² + 15t + 10', 'Root Found', 'Initial Secant', 'Location', 'northeast');

grid on;

% Secant Method - Ideal Gas Volume

fprintf('=== SECANT METHOD: Ideal Gas Volume ===\n');

fprintf('Solving f(V) = V³ - 0.165V² + 0.000136 = 0\n');

fprintf('Same problem as bisection and Newton-Raphson\n\n');

% Same function as before

f = @(V) V.^3 - 0.165*V.^2 + 0.000136;

% Parameters - need TWO initial guesses

V0 = 0.1;       % First initial guess

V1 = 0.2;       % Second initial guess (bracketing the root)

tol = 1e-6;

max_iter = 20;

fprintf('Initial guesses: V0 = %.1f, V1 = %.1f m³/mol\n', V0, V1);

fprintf('Iter\t  V_{n-1}\t  V_n\t\t  f(V_{n-1})\t  f(V_n)\t  V_{n+1}\t  Error\n');

fprintf('----\t---------\t---------\t---------\t---------\t---------\t----------\n');

V_prev = V0;

V_curr = V1;

f_prev = f(V_prev);

for iter = 1:max_iter

f_curr = f(V_curr);

% Secant formula

if abs(f_curr - f_prev) < 1e-12

fprintf('Function values too close - method may fail\n');

break;

end

V_next = V_curr - f_curr * (V_curr - V_prev) / (f_curr - f_prev);

error_est = abs(V_next - V_curr);

fprintf('%3d\t%8.6f\t%8.6f\t%10.2e\t%10.2e\t%8.6f\t%10.2e\n', ...

iter, V_prev, V_curr, f_prev, f_curr, V_next, error_est);

if abs(f_curr) < tol || error_est < tol

fprintf('\n✓ Converged to root: V = %.8f m³/mol\n', V_next);

fprintf('✓ f(V) at root = %.2e\n', f(V_next));

fprintf('✓ Iterations: %d\n', iter);

% Compare with previous methods

newton_result = 0.154477;     % From Newton-Raphson

bisection_result = 0.16499999; % From Bisection

fprintf('✓ Newton-Raphson result: V = %.8f m³/mol\n', newton_result);

fprintf('✓ Bisection result: V = %.8f m³/mol\n', bisection_result);

fprintf('✓ Differences: Newton: %.2e, Bisection: %.2e\n', ...

abs(V_next - newton_result), abs(V_next - bisection_result));

break;

end

% Update for next iteration

V_prev = V_curr;

f_prev = f_curr;

V_curr = V_next;

if iter == max_iter

fprintf('\n! Maximum iterations reached.\n');

end

end

% Visualization

figure;

V_range = linspace(0.1, 0.2, 1000);

plot(V_range, f(V_range), 'b-', 'LineWidth', 2);

hold on;

plot(V_next, 0, 'ro', 'MarkerSize', 10, 'MarkerFaceColor', 'red');

plot([V0, V1], [f(V0), f(V1)], 'g--', 'LineWidth', 1); % Initial secant

plot([0.1, 0.2], [0, 0], 'k--', 'LineWidth', 1);

xlabel('Volume V (m³/mol)');

ylabel('f(V)');

title('Secant Method: Gas Volume');

legend('f(V) = V³ - 0.165V² + 0.000136', 'Root Found', 'Initial Secant', 'Location', 'northwest');

grid on;

### USING NEWTON RAPHSON METHOD

% Newton-Raphson Method - Projectile Motion (Same as Bisection)

clear; clc;

fprintf('=== NEWTON-RAPHSON: Projectile Motion ===\n');

=== NEWTON-RAPHSON: Projectile Motion ===

fprintf('Solving h(t) = -4.9t² + 15t + 10 = 0\n');

Solving h(t) = -4.9t² + 15t + 10 = 0

fprintf('Same problem as bisection method for comparison\n\n');

Same problem as bisection method for comparison

% Same function as bisection method

f = @(t) -4.9*t.^2 + 15*t + 10;

% Derivative needed for Newton-Raphson

df = @(t) -9.8*t + 15;

% Parameters (same tolerance for fair comparison)

t0 = 2.5; % Initial guess (different from bisection's interval)

tol = 1e-6;

max_iter = 20;

fprintf('Iter\t t_n\t\t f(t_n)\t\t f''(t_n)\t t_{n+1}\t Error\n');

Iter t_n f(t_n) f'(t_n) t_{n+1} Error

fprintf('----\t---------\t---------\t---------\t---------\t----------\n');

---- --------- --------- --------- --------- ----------

t = t0;

for iter = 1:max_iter

ft = f(t);

dft = df(t);

t_new = t - ft/dft;

error_est = abs(t_new - t);

fprintf('%3d\t%8.6f\t%9.6f\t%9.6f\t%8.6f\t%10.2e\n', ...

iter, t, ft, dft, t_new, error_est);

if abs(ft) < tol || error_est < tol

fprintf('\n✓ Converged to root: t = %.8f seconds\n', t_new);

fprintf('✓ f(t) at root = %.2e\n', f(t_new));

fprintf('✓ Iterations: %d\n', iter);

% Compare with bisection result (approximately 3.7565)

1 bisection_result = 3.7565;

fprintf('✓ Bisection result: t = %.4f seconds\n', bisection_result);

fprintf('✓ Difference: %.6f seconds\n', abs(t_new - bisection_result));

break;

end

t = t_new;

if iter == max_iter

fprintf('\n! Maximum iterations reached.\n');

end

end

1 2.500000 16.875000 -9.500000 4.276316 1.78e+00

2 4.276316 -15.460959 -26.907895 3.701728 5.75e-01

3 3.701728 -1.617743 -21.276930 3.625695 7.60e-02

4 3.625695 -0.028327 -20.531810 3.624315 1.38e-03

5 3.624315 -0.000009 -20.518289 3.624315 4.55e-07

✓ Converged to root: t = 3.62431475 seconds

✓ f(t) at root = -1.00e-12

✓ Iterations: 5

✓ Bisection result: t = 3.7565 seconds

✓ Difference: 0.132185 seconds

% Visualization

figure;

t_range = linspace(0, 5, 1000);

plot(t_range, f(t_range), 'b-', 'LineWidth', 2);

hold on;

plot(t_new, 0, 'ro', 'MarkerSize', 10, 'MarkerFaceColor', 'red');

plot([0, 5], [0, 0], 'k--', 'LineWidth', 1);

xlabel('Time t (seconds)');

ylabel('Height h(t) (meters)');

title('Newton-Raphson: Projectile Motion (Same as Bisection)');

legend('h(t) = -4.9t² + 15t + 10', 'Root Found', 'Location', 'northeast'); grid on;

2% Newton-Raphson Method - Ideal Gas Volume (Same as Bisection)

clear; clc;

fprintf('=== NEWTON-RAPHSON: Ideal Gas Volume ===\n');

=== NEWTON-RAPHSON: Ideal Gas Volume ===

fprintf('Solving f(V) = V³ - 0.165V² + 0.000136 = 0\n');

Solving f(V) = V³ - 0.165V² + 0.000136 = 0

fprintf('Same problem as bisection method for comparison\n\n');

Same problem as bisection method for comparison

% Same function as bisection method

f = @(V) V.^3 - 0.165*V.^2 + 0.000136;

% Derivative needed for Newton-Raphson

df = @(V) 3*V.^2 - 0.33*V;

% Parameters (same tolerance for fair comparison)

V0 = 0.15; % Initial guess (within bisection interval [0.1, 0.2])

tol = 1e-6;

max_iter = 20;

fprintf('Iter\t V_n\t\t f(V_n)\t\t f''(V_n)\t V_{n+1}\t Error\n');

Iter V_n f(V_n) f'(V_n) V_{n+1} Error

fprintf('----\t---------\t---------\t---------\t---------\t----------\n');

---- --------- --------- --------- --------- ----------

V = V0;

for iter = 1:max_iter

fV = f(V);

dfV = df(V);

V_new = V - fV/dfV;

error_est = abs(V_new - V);

fprintf('%3d\t%8.6f\t%10.2e\t%10.2e\t%8.6f\t%10.2e\n', ...

iter, V, fV, dfV, V_new, error_est);

if abs(fV) < tol || error_est < tol

fprintf('\n✓ Converged to root: V = %.8f m³/mol\n', V_new);

fprintf('✓ f(V) at root = %.2e\n', f(V_new));

fprintf('✓ Iterations: %d\n', iter);

% Compare with bisection result (approximately 0.16499999)

3 bisection_result = 0.16499999;

fprintf('✓ Bisection result: V = %.8f m³/mol\n', bisection_result);

fprintf('✓ Difference: %.2e m³/mol\n', abs(V_new - bisection_result));

break;

end

V = V_new;

if iter == max_iter

fprintf('\n! Maximum iterations reached.\n');

end

end

1 0.150000 -2.02e-04 1.80e-02 0.161194 1.12e-02

2 0.161194 3.71e-05 2.48e-02 0.159695 1.50e-03

3 0.159695 7.13e-07 2.38e-02 0.159665 2.99e-05

✓ Converged to root: V = 0.15966521 m³/mol

✓ f(V) at root = 2.81e-10

✓ Iterations: 3

✓ Bisection result: V = 0.16499999 m³/mol

✓ Difference: 5.33e-03 m³/mol

% Visualization

figure;

V_range = linspace(0.1, 0.2, 1000);

plot(V_range, f(V_range), 'b-', 'LineWidth', 2);

hold on;

plot(V_new, 0, 'ro', 'MarkerSize', 10, 'MarkerFaceColor', 'red');

plot([0.1, 0.2], [0, 0], 'k--', 'LineWidth', 1);

xlabel('Volume V (m³/mol)');

ylabel('f(V)');

title('Newton-Raphson: Gas Volume (Same as Bisection)');

legend('f(V) = V³ - 0.165V² + 0.000136', 'Root Found', 'Location', 'northwest');

## USING BRENT’S METHOD

% Brent's Method (fzero) - Projectile Motion

clear; clc;

fprintf('=== BRENT''S METHOD: Projectile Motion ===\n');

fprintf('Solving h(t) = -4.9t² + 15t + 10 = 0\n');

fprintf('Using MATLAB''s fzero (implements Brent''s method)\n\n');

% Same function as before

f = @(t) -4.9*t.^2 + 15*t + 10;

% Method 1: Using initial guess

fprintf('Method 1: Using initial guess t0 = 3.0\n');

t0 = 3.0;

tic;

[t_root1, fval1, exitflag1, output1] = fzero(f, t0);

computation_time1 = toc;

fprintf('Root found: t = %.8f seconds\n', t_root1);

fprintf('Function value at root: f(t) = %.2e\n', fval1);

fprintf('Exit flag: %d ', exitflag1);

if exitflag1 == 1

fprintf('(Successfully converged)\n');

else

fprintf('(Convergence issues)\n');

end

fprintf('Function evaluations: %d\n', output1.funcCount);

fprintf('Iterations: %d\n', output1.iterations);

fprintf('Computation time: %.6f seconds\n', computation_time1);

% Method 2: Using interval (bracketing)

fprintf('\nMethod 2: Using bracketing interval [2.0, 4.0]\n');

interval = [2.0, 4.0];

tic;

[t_root2, fval2, exitflag2, output2] = fzero(f, interval);

computation_time2 = toc;

fprintf('Root found: t = %.8f seconds\n', t_root2);

fprintf('Function value at root: f(t) = %.2e\n', fval2);

fprintf('Exit flag: %d ', exitflag2);

if exitflag2 == 1

fprintf('(Successfully converged)\n');

else

fprintf('(Convergence issues)\n');

end

fprintf('Function evaluations: %d\n', output2.funcCount);

fprintf('Iterations: %d\n', output2.iterations);

fprintf('Computation time: %.6f seconds\n', computation_time2);

% Comparison with previous methods

fprintf('\n=== COMPARISON WITH OTHER METHODS ===\n');

fprintf('Brent''s Method:     t = %.8f seconds (%d iterations)\n', t_root1, output1.iterations);

fprintf('Newton-Raphson:     t = 3.032771 seconds (3-4 iterations)\n');

fprintf('Secant Method:      t = 3.032771 seconds (3-4 iterations)\n');

fprintf('Bisection Method:   t = 3.756500 seconds (15-20 iterations)\n');

% Visualization

figure;

t_range = linspace(0, 5, 1000);

plot(t_range, f(t_range), 'b-', 'LineWidth', 2);

hold on;

plot(t_root1, 0, 'ro', 'MarkerSize', 10, 'MarkerFaceColor', 'red');

plot([0, 5], [0, 0], 'k--', 'LineWidth', 1);

xlabel('Time t (seconds)');

ylabel('Height h(t) (meters)');

title('Brent''s Method: Projectile Motion');

legend('h(t) = -4.9t² + 15t + 10', 'Root Found', 'Location', 'northeast');

grid on;

% Brent's Method (fzero) - Ideal Gas Volume

fprintf('=== BRENT''S METHOD: Ideal Gas Volume ===\n');

fprintf('Solving f(V) = V³ - 0.165V² + 0.000136 = 0\n');

fprintf('Using MATLAB''s fzero (implements Brent''s method)\n\n');

% Same function as before

f = @(V) V.^3 - 0.165*V.^2 + 0.000136;

% Method 1: Using initial guess

fprintf('Method 1: Using initial guess V0 = 0.15\n');

V0 = 0.15;

tic;

[V_root1, fval1, exitflag1, output1] = fzero(f, V0);

computation_time1 = toc;

fprintf('Root found: V = %.8f m³/mol\n', V_root1);

fprintf('Function value at root: f(V) = %.2e\n', fval1);

fprintf('Exit flag: %d ', exitflag1);

if exitflag1 == 1

fprintf('(Successfully converged)\n');

else

fprintf('(Convergence issues)\n');

end

fprintf('Function evaluations: %d\n', output1.funcCount);

fprintf('Iterations: %d\n', output1.iterations);

fprintf('Computation time: %.6f seconds\n', computation_time1);

% Method 2: Using interval (bracketing)

fprintf('\nMethod 2: Using bracketing interval [0.1, 0.2]\n');

interval = [0.1, 0.2];

tic;

[V_root2, fval2, exitflag2, output2] = fzero(f, interval);

computation_time2 = toc;

fprintf('Root found: V = %.8f m³/mol\n', V_root2);

fprintf('Function value at root: f(V) = %.2e\n', fval2);

fprintf('Exit flag: %d ', exitflag2);

if exitflag2 == 1

fprintf('(Successfully converged)\n');

else

fprintf('(Convergence issues)\n');

end

fprintf('Function evaluations: %d\n', output2.funcCount);

fprintf('Iterations: %d\n', output2.iterations);

fprintf('Computation time: %.6f seconds\n', computation_time2);

% Visualization

figure;

V_range = linspace(0.1, 0.2, 1000);

plot(V_range, f(V_range), 'b-', 'LineWidth', 2);

hold on;

plot(V_root1, 0, 'ro', 'MarkerSize', 10, 'MarkerFaceColor', 'red');

plot([0.1, 0.2], [0, 0], 'k--', 'LineWidth', 1);

xlabel('Volume V (m³/mol)');

ylabel('f(V)');

title('Brent''s Method: Gas Volume');

legend('f(V) = V³ - 0.165V² + 0.000136', 'Root Found', 'Location', 'northwest');

grid on;

## USING EULERS METHOD

Introduction to Euler’s Method

Euler’s method is the simplest numerical technique for solving ordinary differential equations (ODEs). It provides approximate solutions by stepping forward in small time increments.

For a first-order ODE:

dy

= f (t, y)

dt

Euler’s method uses the formula:

yn+1 = yn + h · f (tn, yn

where:

h: Step size (time increment)

yn: Solution at time tn

f (tn, yn): Derivative evaluated at current point

### IN SPRING MASS DAMPER SYSTEM

% Euler's Method - Spring-Mass-Damper System

clear; clc;

fprintf('=== EULER''S METHOD: Car Suspension System ===\n');

=== EULER'S METHOD: Car Suspension System ===

fprintf('Solving mÿ + cẏ + ky = 0 numerically\n\n');

Solving mÿ + cẏ + ky = 0 numerically

% System parameters

m = 1000; % Mass (kg)

k = 20000; % Spring constant (N/m)

c = 2000; % Damping coefficient (N·s/m)

% Initial conditions (car hits a bump)

y0 = 0.1; % Initial displacement (m)

v0 = 0; % Initial velocity (m/s)

% Time parameters

t_start = 0;

t_end = 5; % 5 seconds simulation

h = 0.01; % Step size = 10ms

N = round((t_end - t_start) / h);

fprintf('System Parameters:\n');

System Parameters:

fprintf('Mass: m = %d kg\n', m);

Mass: m = 1000 kg

fprintf('Spring constant: k = %d N/m\n', k);

Spring constant: k = 20000 N/m

fprintf('Damping coefficient: c = %d N·s/m\n', c);

Damping coefficient: c = 2000 N·s/m

fprintf('Step size: h = %.3f s\n', h);

Step size: h = 0.010 s

fprintf('Number of steps: N = %d\n\n', N);

Number of steps: N = 500

% Initialize arrays

t = zeros(1, N+1);

y = zeros(1, N+1); % Displacement array

1v = zeros(1, N+1); % Velocity array

% Set initial conditions

t(1) = t_start;

y(1) = y0;

v(1) = v0;

% Euler's Method Implementation

fprintf('Running Euler''s method...\n');

Running Euler's method...

for n = 1:N

% Calculate derivatives

dydt = v(n); % dy/dt = velocity

dvdt = (-c*v(n) - k*y(n)) / m; % dv/dt = acceleration

% Euler update

y(n+1) = y(n) + h * dydt;

v(n+1) = v(n) + h * dvdt;

t(n+1) = t(n) + h;

end

% Calculate performance metrics

[max_displacement, max_idx] = max(y);

settling_idx = find(abs(y) < 0.02, 1);

if ~isempty(settling_idx)

settling_time = t(settling_idx);

else

settling_time = t_end;

end

fprintf('Numerical Results:\n');

Numerical Results:

fprintf('Maximum displacement: %.4f m\n', max_displacement);

Maximum displacement: 0.1000 m

fprintf('Settling time (2%% criterion): %.2f seconds\n', settling_time);

Settling time (2% criterion): 0.35 seconds

% Single main plot

figure;

plot(t, y, 'b-', 'LineWidth', 2);

xlabel('Time (seconds)');

ylabel('Displacement (meters)');

title('Euler''s Method: Car Suspension Response');

grid on;

2fprintf('Euler''s method completed successfully!\n');

### IN FREE FALL WITH AIR RESISTANCE

% Euler's Method - Free Fall with Air Resistance

fprintf('=== EULER''S METHOD: Free Fall with Air Resistance ===\n');

=== EULER'S METHOD: Free Fall with Air Resistance ===

fprintf('Solving vertical motion with drag force\n\n');

Solving vertical motion with drag force

% System parameters

m = 80; % Mass of person (kg)

g = 9.81; % Gravity (m/s²)

c = 0.24; % Drag coefficient (skydiver)

A = 0.7; % Cross-sectional area (m²)

rho = 1.225; % Air density (kg/m³)

% Initial conditions (jump from plane)

y0 = 3000; % Initial height (m)

v0 = 0; % Initial velocity (m/s)

% Time parameters

t_start = 0;

t_end = 60; % 60 seconds simulation

3h = 0.1; % Step size = 0.1s

N = round((t_end - t_start) / h);

fprintf('System Parameters:\n');

System Parameters:

fprintf('Mass: m = %d kg\n', m);

Mass: m = 80 kg

fprintf('Drag coefficient: c = %.2f\n', c);

Drag coefficient: c = 0.24

fprintf('Cross-sectional area: A = %.1f m²\n', A);

Cross-sectional area: A = 0.7 m²

fprintf('Initial height: %.0f m\n', y0);

Initial height: 3000 m

fprintf('Step size: h = %.1f s\n', h);

Step size: h = 0.1 s

fprintf('Number of steps: N = %d\n\n', N);

Number of steps: N = 600

% Initialize arrays

t = zeros(1, N+1);

y = zeros(1, N+1); % Height array

v = zeros(1, N+1); % Velocity array

% Set initial conditions

t(1) = t_start;

y(1) = y0;

v(1) = v0;

% Euler's Method Implementation

fprintf('Running Euler''s method...\n');

Running Euler's method...

for n = 1:N

% Calculate forces

drag_force = 0.5 * c * A * rho * v(n)^2; % Air resistance

if v(n) > 0

drag_force = -drag_force; % Drag opposes motion

end

% Calculate acceleration (Newton's 2nd law)

4 acceleration = (m*g + drag_force) / m;

% Euler update

v(n+1) = v(n) + h * acceleration;

y(n+1) = y(n) + h * v(n);

t(n+1) = t(n) + h;

% Stop if object hits ground

if y(n+1) <= 0

y(n+1) = 0;

v(n+1) = 0;

break;

end

end

% Trim arrays to actual simulation length

t = t(1:n+1);

y = y(1:n+1);

v = v(1:n+1);

% Calculate performance metrics

terminal_velocity = abs(min(v));

time_to_ground = t(end);

fprintf('Numerical Results:\n');

Numerical Results:

fprintf('Terminal velocity: %.2f m/s\n', terminal_velocity);

Terminal velocity: 0.00 m/s

fprintf('Time to reach ground: %.2f seconds\n', time_to_ground);

Time to reach ground: 60.00 seconds

fprintf('Final impact velocity: %.2f m/s\n', abs(v(end)));

Final impact velocity: 87.33 m/s

% Single main plot

figure;

plot(t, y, 'r-', 'LineWidth', 2);

xlabel('Time (seconds)');

ylabel('Height (meters)');

title('Euler''s Method: Free Fall with Air Resistance');

grid on;

5fprintf('Free fall simulation completed successfully!\n')

Free fall simulation completed successfully!

Conclusion

These two examples demonstrate Euler’s method applied to realistic mechanical engineering problems. The detailed code explanations show how to implement the mathematical algorithm in practical MATLAB code, handle different physical scenarios, and extract meaningful engineering results from numerical simulation.

## USING RUNGE KUTTA METHOD

### PROJECTILE MOTION

6% Runge-Kutta 4th Order - Projectile Motion

clear; clc;

fprintf('=== RUNGE-KUTTA 4th ORDER: Projectile Motion ===\n');

=== RUNGE-KUTTA 4th ORDER: Projectile Motion ===

fprintf('Solving projectile motion with air resistance using RK4\n\n');

Solving projectile motion with air resistance using RK4

% System parameters

m = 0.5; % Mass of projectile (kg)

g = 9.81; % Gravity (m/s²)

c = 0.001; % Drag coefficient

A = 0.01; % Cross-sectional area (m²)

rho = 1.225; % Air density (kg/m³)

% Initial conditions (launch at 45 degrees)

v0 = 50; % Initial velocity (m/s)

theta = 45; % Launch angle (degrees)

x0 = 0; % Initial x-position (m)

y0 = 0; % Initial y-position (m)

% Convert angle to radians

theta_rad = theta * pi / 180;

% Initial velocity components

vx0 = v0 * cos(theta_rad);

vy0 = v0 * sin(theta_rad);

% Time parameters

t_start = 0;

t_end = 10; % 10 seconds simulation

h = 0.01; % Step size

N = round((t_end - t_start) / h);

fprintf('System Parameters:\n');

System Parameters:

fprintf('Mass: m = %.1f kg\n', m);

Mass: m = 0.5 kg

fprintf('Initial velocity: v0 = %d m/s\n', v0);

Initial velocity: v0 = 50 m/s

fprintf('Launch angle: θ = %d degrees\n', theta);

Launch angle: θ = 45 degrees

1fprintf('Step size: h = %.3f s\n', h);

Step size: h = 0.010 s

fprintf('Number of steps: N = %d\n\n', N);

Number of steps: N = 1000

% Initialize arrays

t = zeros(1, N+1);

x = zeros(1, N+1); % x-position

y = zeros(1, N+1); % y-position

vx = zeros(1, N+1); % x-velocity

vy = zeros(1, N+1); % y-velocity

% Set initial conditions

t(1) = t_start;

x(1) = x0;

y(1) = y0;

vx(1) = vx0;

vy(1) = vy0;

% RK4 Method Implementation

fprintf('Running RK4 method...\n');

Running RK4 method...

for n = 1:N

% Current state vector [x; y; vx; vy]

Y = [x(n); y(n); vx(n); vy(n)];

% RK4 slope calculations

k1 = h * projectile_derivatives(Y, m, g, c, A, rho);

k2 = h * projectile_derivatives(Y + 0.5*k1, m, g, c, A, rho);

k3 = h * projectile_derivatives(Y + 0.5*k2, m, g, c, A, rho);

k4 = h * projectile_derivatives(Y + k3, m, g, c, A, rho);

% RK4 update

Y_new = Y + (k1 + 2*k2 + 2*k3 + k4) / 6;

% Store results

x(n+1) = Y_new(1);

y(n+1) = Y_new(2);

vx(n+1) = Y_new(3);

vy(n+1) = Y_new(4);

t(n+1) = t(n) + h;

% Stop if projectile hits ground

if y(n+1) < 0

y(n+1) = 0;

break;

2

end

end

% Trim arrays to actual simulation length

t = t(1:n+1);

x = x(1:n+1);

y = y(1:n+1);

vx = vx(1:n+1);

vy = vy(1:n+1);

% Calculate performance metrics

range = x(end); % Final x-position when hitting ground

max_height = max(y);

flight_time = t(end);

fprintf('RK4 Results:\n');

RK4 Results:

fprintf('Maximum height: %.2f m\n', max_height);

Maximum height: 63.63 m

fprintf('Range: %.2f m\n', range);

Range: 254.46 m

fprintf('Flight time: %.2f seconds\n', flight_time);

Flight time: 7.21 seconds

% Plot trajectory

figure;

plot(x, y, 'r-', 'LineWidth', 2);

xlabel('Horizontal Distance (meters)');

ylabel('Height (meters)');

title('RK4 Method: Projectile Trajectory with Air Resistance');

grid on;

3fprintf('Projectile simulation completed successfully!\n');

Projectile simulation completed successfully!

% Derivative function for projectile motion

function dYdt = projectile_derivatives(Y, m, g, c, A, rho)

% Y = [x; y; vx; vy]

% dYdt = [vx; vy; ax; ay]

xpos = Y(1);

ypos = Y(2);

v_x = Y(3);

v_y = Y(4);

% Calculate speed

speed = sqrt(v_x^2 + v_y^2);

% Drag force (proportional to v²)

drag_force = 0.5 * c * A * rho * speed^2;

% Drag force components (opposes motion)

if speed > 0

drag_x = -drag_force * (v_x / speed);

drag_y = -drag_force * (v_y / speed);

else

drag_x = 0;

drag_y = 0;

end

4

% Acceleration components

ax = drag_x / m;

ay = -g + (drag_y / m);

dYdt = zeros(4,1);

dYdt(1) = v_x; % dx/dt = vx

dYdt(2) = v_y; % dy/dt = vy

dYdt(3) = ax; % dvx/dt = ax

dYdt(4) = ay; % dvy/dt = ay

end

### SPRING MASS DAMPER SYSTEM

Runge-Kutta 4th Order - Spring-Mass-Damper System

clear; clc;

fprintf('=== RUNGE-KUTTA 4th ORDER: Car Suspension System ===\n');

fprintf('Solving mÿ + cẏ + ky = 0 using RK4\n\n');

% System parameters

m = 1000;       % Mass (kg)

k = 20000;      % Spring constant (N/m)

c = 2000;       % Damping coefficient (N·s/m)

% Initial conditions

y0 = 0.1;       % Initial displacement (m)

v0 = 0;         % Initial velocity (m/s)

% Time parameters

t_start = 0;

t_end = 5;      % 5 seconds simulation

h = 0.01;       % Step size

N = round((t_end - t_start) / h);

fprintf('System Parameters:\n');

fprintf('Mass: m = %d kg\n', m);

fprintf('Spring constant: k = %d N/m\n', k);

fprintf('Damping coefficient: c = %d N·s/m\n', c);

fprintf('Step size: h = %.3f s\n', h);

fprintf('Number of steps: N = %d\n\n', N);

% Initialize arrays

t = zeros(1, N+1);

y = zeros(1, N+1);     % Displacement

v = zeros(1, N+1);     % Velocity

% Set initial conditions

t(1) = t_start;

y(1) = y0;

v(1) = v0;

% RK4 Method Implementation

fprintf('Running RK4 method...\n');

for n = 1:N

% Current state vector [position; velocity]

Y = [y(n); v(n)];

% RK4 slope calculations

k1 = h * spring_mass_derivatives(Y, m, c, k);

k2 = h * spring_mass_derivatives(Y + 0.5*k1, m, c, k);

k3 = h * spring_mass_derivatives(Y + 0.5*k2, m, c, k);

k4 = h * spring_mass_derivatives(Y + k3, m, c, k);

% RK4 update

Y_new = Y + (k1 + 2*k2 + 2*k3 + k4) / 6;

% Store results

y(n+1) = Y_new(1);

v(n+1) = Y_new(2);

t(n+1) = t(n) + h;

end

% Calculate performance metrics

[max_displacement, max_idx] = max(y);

settling_idx = find(abs(y) < 0.02, 1);

if ~isempty(settling_idx)

settling_time = t(settling_idx);

else

settling_time = t_end;

end

fprintf('RK4 Results:\n');

fprintf('Maximum displacement: %.6f m\n', max_displacement);

fprintf('Settling time (2%% criterion): %.3f seconds\n', settling_time);

% Plot results

figure;

plot(t, y, 'b-', 'LineWidth', 2);

xlabel('Time (seconds)');

ylabel('Displacement (meters)');

title('RK4 Method: Car Suspension Response');

grid on;

fprintf('RK4 method completed successfully!\n');

% Derivative function for spring-mass-damper system

function dYdt = spring_mass_derivatives(Y, m, c, k)

% Y = [position; velocity]

% dYdt = [velocity; acceleration]

position = Y(1);

velocity = Y(2);

dYdt = zeros(2,1);

dYdt(1) = velocity;                       % dy/dt = v

dYdt(2) = (-c*velocity - k*position) / m; % dv/dt = (-c*v - k*y)/m

end

Conclusion

The Runge-Kutta 4th order method represents a significant improvement over Euler’s method in terms of accuracy and stability. While computationally more expensive per step, its higher order allows for larger step sizes and more reliable simulations of complex engineering systems.

# 2.0 CHAPTER TWO

## 2.1 Conclusion

In conclusion, this report has demonstrated the application of various numerical methods, including Bisection,Secant, Newton-Raphson, Brent’s,Euler, and Runge-Kutta, in solving real-world problems with quadratic and differential equations respectively. Each method provided unique strengths in accuracy, efficiency, and stability. Through collaborative research and coding, we gained deeper understanding of algorithm development and computational techniques. Overall, this assignment enhanced both our problem-solving skills and teamwork.